District of Columbia Municipal Regulations


1307  
EXEMPTED ANTENNAS

1307.1
The requirements of Subtitle C §§ 1303 through 1306 shall not apply to any antenna that is: 

(a)
Entirely enclosed within a building, but is not the primary use within the building; 

(b)
Entirely enclosed on all sides by a penthouse or rooftop structure, or an extension of penthouse or rooftop structure walls or screening; this subsection shall not be interpreted to permit a penthouse or rooftop structure in excess of the height limitations;

1307.2
For the purposes of Subtitle C § 1307.1, parapets and the walls of penthouses, and rooftop structures may include an opaque membrane covering a port in front of the antenna that screens the antenna, blends with the wall and allows the antenna to operate.

1307.1 The requirements of Subtitle C §§ 1303 through 1306 do not apply to the following classes of antennas. The maximum number of antennas within a class that may be placed on a building or located on a lot is as follows:

TABLE C § 1307.3:  MAXIMUM NUMBER OF ANTENNAS BY CLASS

	Class of Antenna
	Maximum Number

	Residential type uhf/vhf television and frequency modulation (fm) radio receiving Yagi antenna located on the roof of a principal building, not to exceed 8 ft. horizontally.
	2

	Whip antennas not exceeding 2½ in. in diameter, with a mounted dimension. (Except that no longer than 12 ft. in any direction, and there located on a principal building, shall be no numeric limit on the number of whip antennas that are dedicated to the provision of emergency services to the District of Columbia.)
	2

	Residential type super high frequency antenna located on the roof of a principal building, not to exceed 3 ft. in any dimension, excluding the support element.
	1

	Dish antenna located on the roof of a principal building with a diameter of no more than 4 ft., not taller than 8 ft. as measured from the roof surface on which it is mounted, and set back from the edge of the roof a distance at least equal to its height above the roof. The principal building shall have a height of no less than 25 ft.
	1

	Whip antenna mounted on a vehicle on private property.
	1


1307.2 A proposed antenna which does not comply with the above requirements or numeric limitation set forth in Subtitle C § 1307.3 may be permitted subject to the requirements of Subtitle C §§ 1303 through 1306.

SOURCE: Final Rulemaking published at 63 DCR 2447, 2712 (March 4, 2016 – Part 2); as amended by Final Rulemaking published at 68 DCR 013834 (December 24, 2021).

	
	Zoning Regulations of 2016: 
General Rules
11-C DCMR § 1307

	

	
	
	



