District of Columbia Municipal Regulations


547
SOIL EROSION AND SEDIMENT CONTROL: RESPONSIBLE PERSONNEL

547.1
If a site involves a land disturbance of five thousand square feet (5,000 ft2) or more, the owner of the site and the site manager shall ensure that a responsible person is present or available as this section requires.

547.2
A responsible person shall, while the site is in a phase involving land-disturbing activity, ensure that the activity complies with this chapter by:

(a) Inspecting the site and its erosion and sediment control measures at least once biweekly and after a rainfall event to identify and remedy each potential or actual erosion problem;

(b) Being available to respond to each potential or actual erosion problem identified by construction personnel; and

(c) Being available to speak on site with the Department to remedy each potential or actual erosion problem.

547.3
A responsible person shall be:

(a) Licensed in the District of Columbia as a land surveyor, architect, or civil, environmental, or geotechnical engineer; or 

(b) Certified through a training program that the Department approves, including a course on erosion control provided by another jurisdiction or professional association.

547.4
During construction, the responsible person shall have available on-site proof of professional licensing or of successful completion of a Department-approved training program. 

547.5
A Department-approved training program shall cover the following topics, as demonstrated in the training syllabus: 

(a) 
The detrimental effects of sediment pollution to waterbodies;

(b) 
The benefits of proper and effective erosion and sediment control implementation and maintenance; 

(c) 
The purpose and provisions of the District of Columbia erosion and sediment control laws, rules, and regulations; 

(d) 
A description of sediment as a pollutant; 

(e) 
The process of: 

(1) 
Erosion; 

(2) 
Sediment transport; and 

(3) 
Sediment deposition; 

(f) 
Proper implementation of erosion and sediment control; 

(g) 
Recognition and correction of improperly implemented erosion and sediment controls; 

(h) 
Proper maintenance of erosion and sediment controls; and 

(i) 
Responsibilities of supervisory and enforcement personnel.

SOURCE: Final Rulemaking published at 60 DCR 10640 (July 19, 2013); as amended by Final Rulemaking published at 67 DCR 818 (January 31, 2020).
	
	Water and Sanitation
21 DCMR § 547

	

	
	
	



