District of Columbia Municipal Regulations


207

REAR YARD 

207.1
Except as provided elsewhere in this title, the minimum required rear yard shall be as set forth in this section. 

207.2
The minimum depth of the rear yard shall be two and one-half inches (2.5 in.) per one foot (1 ft.) of vertical distance from the mean finished grade at the middle of the rear of the structure to the highest point of the main roof or parapet wall, but not less than twelve feet (12 ft.).

207.3
Except as required in Subtitle J § 207.5, the rear yard need not be provided below a horizontal plane twenty feet (20 ft.) above the mean finished grade at the middle of the rear of the structure. 

207.4
For that portion of the structure above the horizontal plane, the depth of rear yard may be measured as follows: 

(a)
When the lot abuts an alley, from the center line of the alley to the rear wall of the portion immediately above the plane; or 

(b) When the lot does not abut an alley, from the rear lot line to the rear wall of the portion immediately above the plane.

207.5
Where the rear lot line of a lot abuts or is separated only by an alley from a residential zone as defined by Subtitle A § 101.9 or residential use of a property, a rear yard shall be provided from the ground up, subject to the transition setback requirements of Subtitle J § 210.

207.6
A required rear yard may be within a required transition setback provided all transitional setback conditions are met.

SOURCE: Final Rulemaking published at 63 DCR 2447, 3163 (March 4, 2016 – Part 2); as amended by Final Rulemaking published at 64 DCR 22 (January 6, 2017); as amended by Final Rulemaking published at 70 DCR 011297 (August 25, 2023).
	
	Zoning Regulations of 2016: 
Production, Distribution, and Repair (PDR) Zones
11-J DCMR § 207

	

	
	
	



