District of Columbia Municipal Regulations


1101

HEIGHT
1101.1
Except as provided in Subtitle D §§ 203.3 through 203.6, in the R-1B/GT and R-3/GT zones, the maximum permitted height and number of stories for buildings shall be as follows:

(a) Thirty-five feet (35 ft.) and three (3) stories, not including a penthouse or rooftop structure; or 

(b) Forty feet (40 ft.), not including a penthouse or rooftop structure, only if a property adjacent on either side has a building height of forty feet (40 ft.) or greater.

1101.2
In the R-1B/GT and R-3/GT zones, the maximum height of a building shall be measured to the highest point of the roof or a parapet which is not a required firewall. 

1101.3
In R-1B/GT and R-3/GT zones, a two (2) or more story addition to a principal building which has an existing second story side yard shall not exceed the vertical plane of that yard for the length of the second story addition. 

1101.4
In R-1B/GT and R-3/GT zones, any parapet, pergola, railing, or similar rooftop structure, or penthouse shall not exceed the permitted building height by more than four feet (4 ft.). 

SOURCE: Final Rulemaking published at 70 DCR 011297 (August 25, 2023).
	
	Zoning Regulations of  2016: 
Residential House (R) Zones
11-D DCMR § 1101

	

	
	
	



