District of Columbia Municipal Regulations


544
SOIL EROSION AND SEDIMENT CONTROL: ROADWAY PROJECTS

544.1
Rough graded rights-of-way awaiting installation of utilities or pavement shall be protected by the installation of:

(a) Interceptor dikes across rights-of-way so located as to limit roadway grade to a length between dikes of not more than five hundred feet (500 ft); or

(b) Alternative controls that are recommended by a Professional Engineer (PE) licensed in the District of Columbia and that are approved by the Department.

544.2
Temporary diversion dikes and flumes, or alternative controls that are recommended by a PE licensed in the District of Columbia and that are approved by the Department, shall be used to carry runoff down cut-and-fill slopes to an outlet approved by the Department as part of the soil erosion and sediment control plan.

544.3
A permanent drainage structure, including diversions at top-of-slope cuts and diversions to lead runoff to a storm sewer or other suitable outlet, shall be installed at the completion of rough grading, unless the Department approves an alternative that has been recommended by a PE licensed in the District of Columbia.

SOURCE: 8 DCRR, Health Regulations (1965 Edition); as amended by the Department of Environmental Services’ Handbook for Erosion and Sediment Control (February 1977); as amended by Final Rulemaking published at 25 DCR 1033 (July 28, 1978); incorporating text of Proposed Rulemaking published at 24 DCR 10522 (June 9, 1978); as amended by Final Rulemaking published at 60 DCR 10640 (July 19, 2013).
	
	Water and Sanitation
21 DCMR § 544

	

	
	
	



