District of Columbia Municipal Regulations


AF101

SCOPE
Strike Section AF101.1 in Appendix F of the International Residential Code in its entirety and insert new Section AF101.1 in Appendix F of the Residential Code in its place to read as follows:

AF101.1 General.  Radon-resistant construction built in accordance with Section AF103 is required for new construction. These requirements are intended to provide a passive means of resisting radon gas entry and prepare the dwelling for post-construction radon mitigation, if necessary (see Figure AF102). Additions and Level 3 alterations should test their homes for radon and make radon testing results available upon request by the code official at permitting.  If the testing results show 4 or more picocuries per liter (pCi/L) of radon, a radon system compliant with AF103 shall be installed for the work area.
Exception: Additions less than 100 square feet.

[image: image1.png]TYPICAL SUBSLAB DEPRESSURIZATION PASSIVE RADON SYSTEM VENTED

FASENE AAbON SveTEN THRoua S
s i/
* o
ame xme
* o
Lie area Lne Area
sroe annoe
Srve N e o
BASEMENT i H BASEMENT
POLYETHYLENE S H SUMP POLYETHYLENE
o S ey TR SESS
o r
PASSIVE RADON SYSTEM UsiNG SUBMEMBRANE DEPRESSURZATION SYSTEM
BRAR TLE CooF G GPANL SPAGE
* *
ame
&
* ATTIC
Lvie area
- * Lmearea |8
SWE
assenent :
PoLvEnHVLENE
ey SHEETING ol
Rt I csh 2o8 | CRAWL SPACE
CeRAEL ,- penronuTED
L o e | e R
a1 Sl |
DRAIN TILE LOOP 3l }




FIGURE AF101
RADON-RESISTANT CONSTRUCTION DETAILS FOR FOUR FOUNDATION TYPES
SOURCE: Final Rulemaking published at 67 DCR 6033 (May 29, 2020 – Part 2).
The District of Columbia Residential Code (2017), referred to as the “Residential Code,” consists of the 2015 edition of the International Residential Code (International Residential Code), published by the International Code Council (ICC), as amended by the Residential Code Supplement of 2017 (12-B DCMR). The International Residential Code is copyrighted by the ICC and therefore is not republished here. However, a copy of the text may be obtained at https://codes.iccsafe.org/public/document/IRC2015.

	
	Residential Code Supplement of 2017
12-B DCMR § AF101

	

	
	
	



