District of Columbia Municipal Regulations


525
STORMWATER MANAGEMENT: SHARED BEST MANAGEMENT PRACTICE

525.1
A Shared Best Management Practice (S-BMP) may, upon approval by the Department:
(a) 
Provide stormwater management for a major regulated project in satisfaction of an on-site stormwater management requirement of that project; and

(b) 
Be eligible for Department certification of a Stormwater Retention Credit (SRC).

525.2
A Department-approved S-BMP may provide stormwater management for a nearby property if:

(a) Stormwater flow from the nearby property is directly conveyed to the S-BMP; and

(b) The S-BMP has sufficient capacity.

525.3
To obtain Department approval of the use of an existing S-BMP, a major regulated project shall show how each requirement of the project will be met by the S-BMP, including: 

(a) Submit an as-built Stormwater Management Plan (SWMP) for the S-BMP that is accurate as of the time of submittal;

(b) Prove sufficient capacity of the S-BMP;

(c) Demonstrate the adequacy of each stormwater conveyance from the major regulated project to the S-BMP; and 

(d) Show each drainage area conveying stormwater into the S-BMP from the major regulated project.

525.4 To obtain Department approval of the use of a proposed S-BMP, a major regulated project shall show how each requirement of the project will be met by the S-BMP, including: 

(a) Submit a Department-approved SWMP for the S-BMP;

(b) Prove sufficient capacity of the S-BMP;

(c) Demonstrate the adequacy of each stormwater conveyance from the major regulated project to the S-BMP; and

(d) Show each drainage area conveying stormwater into the S-BMP from the major regulated project.

525.5
A major regulated project that uses a S-BMP to meet a requirement shall not pass the Department’s final inspection until the S-BMP passes the Department’s final inspection and is operational.

525.6
After an alteration to a S-BMP to provide stormwater management for another site, the site with the S-BMP shall:

(a) Pass the Department’s inspection; and

(b) Submit an as-built SWMP, showing each area draining into the S-BMP and the means of conveyance. 

525.7
The Department may certify a SRC for a S-BMP if the S-BMP meets each requirement for certification. 

525.8
A site with a S-BMP that provides a volume of stormwater management to satisfy an on-site requirement of a major regulated project shall be responsible for maintenance of the S-BMP capacity to manage that volume and shall record that responsibility in a declaration of covenants. 

525.9
If the Department determines that a S-BMP has ceased satisfying an on-site retention requirement for a site that underwent a major regulated project, the site shall be responsible for retaining the required volume on site or via use of off-site retention.

SOURCE: Final Rulemaking published at 60 DCR 10640 (July 19, 2013).
	
	Water and Sanitation
21 DCMR § 525

	

	
	
	



