District of Columbia Municipal Regulations


1101

GENERAL DEVELOPMENT STANDARDS (BF)
1101.1
The development standards of this section and Subtitle K § 1102 apply to all BF zones except as modified for a specific zone, in which case the modified zone-specific standard shall apply. When only a portion of a development standard is modified the remaining portions of the development standards shall still apply.
1101.2
A court is not required in a BF zone, but where provided, it shall have the following minimum dimensions:

TABLE K § 1101.2: MINIMUM COURT DIMENSIONS

	Type of Structure
	Open Court

Minimum Width
	Closed Court Minimum Width
	Closed Court

Minimum Area

	Detached, Semi-detached, 

Row Dwellings, 

and Flats
	N/A
	N/A
	N/A

	Residential, more than 3 units
	2.5 inches per 

foot of height of court;

10 ft. minimum
	2.5 inches per 

foot of height of court;

12 ft. minimum
	Twice the square of the required closed court width;

250 square ft. minimum

	Non-residential and Lodging
	2.5 inches per

foot of height of court;

6 ft. minimum
	2.5 inches per

foot of height of court;

10 ft. minimum
	Twice the square of the required closed court width;

250 square ft. minimum


SOURCE: Final Rulemaking published at 68 DCR 014144 (December 31, 2021).
	
	Zoning Regulations of 2016: 
Special Purpose Zones
11-K DCMR § 1101

	

	
	
	



