District of Columbia Municipal Regulations


208

SIDE YARD 

208.1
Except as provided elsewhere in this title, the minimum side yard shall be as set forth in this section. 

208.2
Side yards for a detached or semi-detached building containing one (1) or two (2) dwelling units shall be a minimum of eight feet (8 ft.). No side yards shall be required for a row building containing one (1) or two (2) dwelling units. 

208.3

Except as provided in Subtitle F § 208.2, the following side yard rules apply: 

(a)
In any of the RA-1 zones, one (1) side yard shall be provided unless the building is a multiple dwelling that contains three (3) or more dwelling units per floor, in which case two (2) side yards shall be provided; in either case such side yards shall have the minimum distance equal to three inches (3 in.) per foot of building height but not less than eight feet (8 ft.); and 

(b)
In any of the RA-2, RA-3, RA-4, and RA-5 zones, no side yards are required; however, if a side yard is provided, it shall be a minimum of four feet (4 ft.). 

208.4
A side yard shall not be required along a side street abutting a corner lot in any of the RA-1, RA-2, RA-3, RA-4, and RA-5 zones.

208.5
Existing conforming side yards shall not be reduced to a nonconforming width or eliminated.

208.6
In the case of a building with a non-conforming side yard, an extension or addition may be made to the building; provided, that the width of the existing side yard shall not be reduced or eliminated; and provided further, that the width of the side yard adjacent to the extension or addition shall be a minimum of three feet (3 ft.).

208.7
In the case of a lot proposed to be used by a public library or public recreation and community center where a side lot line abuts or adjoins a public open space, recreation area, or reservation, no side yard shall be required.

SOURCE: Final Rulemaking published at 70 DCR 011297 (August 25, 2023).
	
	Zoning Regulations of 2016: 
Residential Apartment (RA) Zones
11-F DCMR § 208

	

	
	
	



