District of Columbia Municipal Regulations


540  
HEIGHT (D-5)

540.1 The maximum permitted building height, not including the penthouse or rooftop structure, in the D-5 zone shall be as follows:

	Street Right-of-Way Width
	Maximum Permitted Building Height, Not Including Penthouse or Rooftop Structure

	Greater than or equal to 110 ft.
	130 feet

	Less than 110 ft. but greater than or equal to one 100 ft.
	120 feet

	Less than 100 ft. but greater than or equal to 90 ft.
	110 feet

	Less than 90 ft.
	No taller than the width of the street right of way, plus 20 feet


540.2 A building on a lot in the D-5 zone with frontage on one (1) or more of the following designated street segments shall also be subject to the height regulations governing the sub-area in which the segment is located:

(a) The Massachusetts Avenue Corridor and Mount Vernon Square Sub-Area regulations in Subtitle I § 610 apply to properties fronting on Massachusetts Avenue, or Mount Vernon Square N.W., between 3rd and 13th Streets, N.W.; and

(b) The Blagden Alley Residential Transition Sub-Area regulations in Subtitle I § 613 apply to properties fronting on M Street, N.W. between 9th and 10th Streets, N.W.

540.3 The maximum permitted building height, not including the penthouse or rooftop structure, in the D-5 zone shall be limited to ninety feet (90 ft.) on the portion of the site occupied by a historic landmark or a contributing building within a historic district.
540.4 A building located west of 20th Street shall be subject to the upper story setback regulations in Subtitle I § 201.6. 

540.5 The maximum permitted height of a penthouse or rooftop structure shall be twenty feet (20 ft.). The maximum permitted number of stories for penthouse habitable space shall be one (1) plus a mezzanine, except a second story shall be permitted for penthouse mechanical space.
SOURCE: Final Rulemaking published at 63 DCR 2447, 3041 (March 4, 2016 – Part 2); as amended by Final Rulemaking published at 63 DCR 10932 (August 26, 2016); as amended by Final Rulemaking published at 68 DCR 013834 (December 24, 2021).
	
	Zoning Regulations of 2016: 
Downtown (D) Zones
11-I DCMR § 540

	

	
	
	



