District of Columbia Municipal Regulations


546
SOIL EROSION AND SEDIMENT CONTROL: UNDERGROUND UTILITIES

546.1
If the land-disturbing activity involves work on an underground utility, the site shall comply with the following requirements:

(a) 
No more than five hundred linear feet (500 ft) of trench shall be open at any one time;

(b)
All excavated material shall be placed on the uphill side of a trench;

(c) 
Interim or permanent stabilization shall be installed upon completion of refilling; and

(d) 
When natural or artificial grass filter strips are used to collect sediment from excavated material, mulches and matting shall be used in order to minimize erosion of these materials. 

SOURCE: Final Rulemaking published at 60 DCR 10640 (July 19, 2013).
	
	Water and Sanitation
21 DCMR § 546

	

	
	
	



