699
DEFINITIONS AND ABBREVIATIONS

699.1
The meanings ascribed to the definitions and abbreviations appearing in §§ 199.1 and 199.2 respectively of Chapter 1 shall apply to the terms and abbreviations in this chapter.

	APPENDIX 6-1


	
	Process Maximum

	
	Process Maximum


	Process Weight*
Per Hour in

Pounds


	Weight of Parti-
culate Discharge Per

Hour in Pounds


	Process Weight*
Per Hour in

Pounds


	Process Maximum
Weight of Parti-

culate Discharge Per

Hour in Pounds



	50
	.24
	3000
	5.10

	100
	.46
	3100
	5.18

	150
	.66
	3200
	5.27

	200
	.85
	3300
	5.36

	250
	1.03
	3400
	5.44

	300
	1.20
	3500
	5.52

	350
	1.35
	3600
	5.61

	400
	1.50
	3700
	5.69

	450
	1.63
	3800
	5.77

	500
	1.77
	3900
	5.85

	550
	1.89
	4000
	5.93

	600
	2.01
	4100
	6.01

	650
	2.12
	4200
	6.08

	700
	2.24
	4300
	6.15

	750
	2.34
	4400
	6.22

	800
	2.43
	4500
	6.30

	850
	2.53
	4600
	6.37

	900
	2.62
	4700
	6.45

	950
	2.72
	4800
	6.52

	1000
	2.80
	4900
	6.60

	1100
	2.97
	5000
	6.67

	1200
	3.12
	5500
	7.03

	1300
	3.26
	6000
	7.37

	1400
	3.40
	6500
	7.71

	1500
	3.54
	7000
	8.05

	1600
	3.66
	7500
	8.39

	1700
	3.79
	8000
	8.71

	1800
	3.91
	8500
	9.03

	1900
	4.03
	9000
	9.36

	2000
	4.14
	9500
	9.67

	2100
	4.24
	10000
	10.00

	2200
	4.34
	11000
	10.63

	2300
	4.44
	12000
	11.28

	2400
	4.55
	13000
	11.89

	2500
	4.64
	14000
	12.50

	2600
	4.74
	15000
	13.13

	2700
	4 84
	16000
	13.74

	2800
	4.92
	17000
	14.36

	2900
	5.02
	18000
	14.97

	19000
	15.58
	50000
	34.30

	20000
	16.19
	60000
	40.00

	30000
	22.22
	or more
	

	40000
	28.30
	
	


*Where the process weight per hour falls between two (2) values in the table, the maximum weight per hour shall be determined by linear interpolation:


SOURCE: Section 3 of the District of Columbia Air Pollution Control Act of 1984, D.C. Law 5-165, § 603.1, 32 DCR 565, 650 (February 1, 1985).

