1722
STREETWALL DESIGN REQUIREMENTS (DD/MVT)

1722.1
Except for a building located in the Principal Intersection Area, as described in § 1723.1, or a building located on Lot 158 in Square 515 for which a building permit has been received within nine (9) months prior to or after [the effective date of this section], or any portions of a building that existed on the [effective date of this section] that is subsequently designated a historic landmark or is included within a historic district, this Section together with § 1701.3, shall apply to each building with frontage on:

(a)
K Street in Squares 451, 483, 484, 484W, 515, or 516;

(b)
5th Street in Squares 483, 484, 515 or 516;

(c)
6th Street in Squares 484 or 484W, if that frontage is within thirty-six feet (36 ft.) of the right-of way of K Street; and

(d)
4th Street in Square 515 or 516, if that frontage is within thirty-six feet (36 ft.) of the right-of way of K Street.

1722.2
Each building shall devote not less than fifty percent (50%) of the surface area of the streetwall(s) at the ground level of each building to display windows with clear or clear and/or low-emissivity glass, except for decorative or architectural accent, and to entrances to commercial uses or to the building.

1722.3
Each building shall devote not less than fifty percent (50%) of the surface area of the streetwall between twelve (12) feet and fourteen (14) feet above grade to clear or clear and/or low-emissivity glass.

1722.4
Each building shall be designed so as not to preclude an entrance every forty (40) feet on average for the linear frontage of the building; excluding vehicular entrances, but including entrance to ground floor uses and any main lobby.

1722.5
The ground floor level of each new building or building addition shall have a minimum clear floor-to-ceiling height of fourteen (14) feet for a distance of at least thirty-six (36) feet perpendicular to the ground floor's front building line.


SOURCE: Final Rulemaking published at 53 DCR 7860 (September 29, 2006).

