4013
RIGHT TURN ON RED

4013.1
Right turns after coming to a full stop and yielding right-of-way to pedestrians and other vehicles shall be allowed when facing red traffic control signals, except at locations listed in this section.
4013.2
Right turns by vehicles facing red traffic signals shall be prohibited at all times on all days of the week at the intersections listed in this subsection.

4013.3
Right turns by vehicles facing a red traffic signal shall be prohibited during the hours from 7:00 a.m. to 7:00 p.m., on all days of the week at the intersections listed in this subsection.

4013.4
Right turns on red will be prohibited at all intersection approaches with one (1) or more of the following characteristics:

(a)
Sight distance of vehicles approaching from the left is less than one of the following minimums:
	Cross Street Speed Limit
	Minimum Sight Distance
	Cross Street Speed Limit
	Minimum Sight Distance

	20 mph
	120 feet
	40 mph
	270 feet

	25 mph
	150 feet
	45 mph
	320 feet

	30 mph
	190 feet
	50 mph
	360 feet

	35 mph
	220 feet
	55 mph
	410 feet


(b)
The intersection has geometrical or operational characteristics which may result in unexpected vehicle or pedestrian conflicts;

(c)
There is significant crossing activity by children, elderly, or handicapped persons;

(d)
Significant pedestrian conflicts are expected due to a high volume of pedestrian crossings; and

(e)
There is an exclusive pedestrian “WALK” signal at the intersection.


SOURCE: 17 DCRR § 40.111, Special Edition (September 28, 1970); as amended by Final Rulemaking published at 25 DCR 11028 (June 29, 1979), incorporating text of Proposed Rulemaking published at 25 DCR 9742, 9743 (April27, 1979); as amended by Final Rulemaking published at 28 DCR 3205 (July 17, 1981), incorporating text of Proposed Rulemaking published at 28 DCR 1624 (April 10, 1981); and by Final Rulemaking published at 28 DCR 4479 (October 16, 1981), incorporating text of Proposed Rulemaking published et 28 DCR 3237. 3238 (July 17, 1981).

