1533
FUNCTIONALITY ─ VENDING MACHINES, LIQUID WASTE PRODUCTS  

1533.1
Vending machines designed to store beverages that are packaged in containers made from paper products shall be equipped with diversion devices and retention pans or drains for container leakage.  

1533.2
Vending machines that dispense liquid food in bulk shall be:   

(a)
Provided with an internally mounted waste receptacle for the collection of 
drip, spillage, overflow, or other internal wastes; and

(b)
Equipped with an automatic shutoff device that will place the machine out 
of operation before the waste receptacle overflows.  

1533.3
Shutoff devices specified in § 1533.2(b) shall prevent water or liquid food from continuously running if there is a failure of a flow control device in the water or liquid food system or waste accumulation that could lead to overflow of the waste receptacle.  

SOURCE: Final Rulemaking published at 50 DCR 4394 (June 6, 2003); as amended by Final Rulemaking published at 59 DCR 13690 (November 30, 2012).
