713

NUMBERS AND CAPACITIES ─ BACKFLOW PREVENTION DEVICE, WHEN REQUIRED*  
713.1

A plumbing system shall be installed to preclude backflow of a solid, liquid, or gas contaminant into the water supply system, including on a hose bibb if a hose is attached or on a hose bibb if a hose is not attached and backflow prevention is required by the Plumbing Code by:  

(a)
Providing an air gap as specified in section 710; or
(b)
Installing an approved backflow prevention device as specified in section 711.  
Source: Final Rulemaking published at 56 DCR 7975 (October 9, 2009).
