1601
EQUIPMENT ─ MANUAL WAREWASHING, SINK COMPARTMENT REQUIREMENTS  

1601.1
A sink with at least three (3) compartments shall be provided for manually washing, rinsing, and sanitizing equipment and utensils, except as specified in § 1601.3.Pf
1601.2
Sink compartments shall be large enough to accommodate immersion of the largest equipment and utensils. If equipment or utensils are too large for the warewashing sink, a warewashing machine or alternative equipment as specified in § 1601.3 shall be used. Pf  

1601.3
Alternative manual warewashing equipment may be used when there are special cleaning needs or constraints and its use is approved. Alternative manual warewashing equipment may include:  

(a)
High-pressure detergent sprayers;

(b)
Low- or line-pressure spray detergent foamers;

(c)
Other task-specific cleaning equipment;

(d)
Brushes or other implements;

(e)
Two (2)-compartment sinks as specified in §§ 1601.4 and 1601.5; or

(f)
Receptacles that substitute for the compartments of a multi-compartment 
sink.  

1601.4
Before a two (2)-compartment sink is used:  

(a)
The licensee shall have its use approved; and

(b)
The licensee shall limit the number of kitchenware items cleaned and 
sanitized in the two (2)-compartment sink, and shall limit warewashing to 
batch operations for cleaning kitchenware such as between cutting one 
type of raw meat and another or cleanup at the end of a shift, and shall:    

(1)
Make up the cleaning and sanitizing solutions immediately before 
use and drain them immediately after use, and 

(2)
Use a detergent-sanitizer to sanitize and apply the detergent-
sanitizer in accordance with the manufacturer’s label instructions 
and as specified in § 1814; or 

(3)
Use a hot water sanitization immersion step as specified in § 
1909.1(c).    

1601.5
A two (2)‑compartment sink shall not be used for warewashing operations where cleaning and sanitizing solutions are used for a continuous or intermittent flow of kitchenware or tableware in an ongoing warewashing process.  

SOURCE: Final Rulemaking published at 50 DCR 4394 (June 6, 2003); as amended by Final Rulemaking published at 59 DCR 13690 (November 30, 2012).
