1701
INSTALLATION ─ FIXED EQUIPMENT, SPACING OR SEALING  

1701.1
Equipment that is fixed because it is not easily movable shall be installed so that it is:  

(a)
Spaced to allow access for cleaning along the sides, behind, and above the 
equipment;

(b)
Spaced from adjoining equipment, walls, and ceilings a distance of not 
more than one millimeter (1 mm) or one thirty-second of an inch (1/32 
in.); or

(c)
Sealed to adjoining equipment or walls, if the equipment is exposed to 
spillage or seepage.  

1701.2
Counter-mounted equipment that is not easily movable shall be installed to allow cleaning of the equipment and areas underneath and around the equipment by being:  

(a)
Sealed; or

(b)
Elevated on legs as specified in § 1702.4.
SOURCE: Final Rulemaking published at 50 DCR 4394 (June 6, 2003); as amended by Final Rulemaking published at 59 DCR 13690 (November 30, 2012).
