District of Columbia Municipal Regulations


1601
BONUS DENSITY AND HEIGHT (CG)

1601.1
CG Overlay developments subject to the set-aside requirements of Chapter 26 Inclusionary Zoning may use the FAR, Height and Lot Occupancy in the following table as the basis of calculating the set-aside requirements of § 2603:

	
	
	IZ Bonus
	

	Base Zone
	FAR
	Lot

Occupancy
	Maximum

Height

	Capitol Gateway
	
	
	

	C-2-C
	7.2
	80%
	110

	C-3-C
	7.8
	100%
	90

	CR
	7.2
	80%
	100

	W-1
	3.0
	80%
	50

	W-2
	4.8
	75%
	80

	W-3
	7.2
	75%
	100


Bonus density achieved via §§ 1601.2 or 1601.4 does not add to the set-aside requirements of § 2603.

1601.2
In the CG/CR and CG/W-3 Districts, a building or combined lot development shall be allowed a maximum density of 8.2 FAR; provided that the additional 1.0 FAR in excess of § 1601.1 FAR shall be devoted solely to residential uses, which, for the purposes of this subsection, does not include hotel uses.

1601.3
For the purpose of accommodating bonus density as authorized by § 1601.1, the maximum permitted building height shall be that permitted by the Act to Regulate the Height of Buildings in the District of Columbia, approved June 1, 1910 (36 Stat. 452; D.C. Official Code §§ 6-601.01 to 6-601.09), as amended; provided that in Squares 601, 656, and 657 those lots abutting or separated only by a street or alley from residentially zoned property shall provide a one-to-one (1:1) building setback for any part of a building that exceeds ninety (90) feet in height on the side abutting the residential zone.

1601.4
In the CG/W-1 District, a building or combined lot development shall be allowed a maximum density of 4.0 FAR and a maximum height of fifty- five (55) feet to accommodate the additional density. The additional 1.0 FAR in excess of §1601.1 shall be devoted solely to residential uses unless the building or the combined lot development includes at least 2.0 FAR of residential uses, in which case the additional 1.0 FAR may be devoted to any permitted use in the W-1 zone. For the purposes of this subsection, the term "residential uses" does not include hotel uses.

1601.5
In the CG/W-2 District, the Zoning Commission may grant additional density to lots as part of the review and approval process applicable to that area, in the manner set forth in §§ 1603.5 and 1603.6.

1601.6
A building that qualifies as a Capitol South Receiving Zone site under § 1709.18 shall be subject to the maximum height and bulk limits of § 1709.21 of this title.

1601.7
The provisions of § 411 shall apply to penthouses in the CG Overlay.  

SOURCE: Final Rulemaking published at 52 DCR 72 (January 7, 2005); as amended by Final Rulemaking published at 55 DCR 2614 (March 14, 2008); as amended by Final Rulemaking published at 63 DCR 390 (January 8, 2016).
	
	Zoning
11 DCMR § 1601

	

	
	
	



