District of Columbia Municipal Regulations


701  
DENSITY – FLOOR AREA RATIO (FAR) (RC)

701.1 Except as provided in other provisions of this chapter, the maximum permitted floor area ratio (FAR) in a RC zone shall be as set forth in the following table:

TABLE K § 701.1:  MAXIMUM PERMITTED FAR

	Zone
	Use
	Maximum FAR

	RC-1


	Public Library
	2.0

	
	All Other
	1.8

	
	All Other with IZ
	2.16

	RC-2


	Total
	2.5

	
	Total with IZ
	3.0

	
	Non-Residential
	1.5 for lots larger than 10,000 sq. ft.

	RC-3


	Total
	3.5

	
	Total with IZ
	4.2

	
	Non-Residential
	1.5 for lots larger than 10,000 sq. ft.


701.2 In the RC-2 and RC-3 zones, an existing building on a lot with an area ten thousand square feet (10,000 sq. ft.) or less may have a maximum FAR of 2.0 for non-residential uses, provided the uses are located in the ground story, and the story directly above the ground story and the use shall not include eating or drinking uses. 

701.3 Inclusionary Zoning requirements for the RC zones are as specified in Subtitle C, Chapter 10.

SOURCE: Final Rulemaking published at 63 DCR 2447, 3176 (March 4, 2016 – Part 2).
	
	Zoning Regulations of  2016: 
Special Purpose Zones
11-K DCMR § 701

	

	
	
	



