1515
FUNCTIONALITY ─ DISPENSING EQUIPMENT, PROTECTION OF EQUIPMENT AND FOOD  

1515.1
In equipment that dispenses or vends liquid food or ice in unpackaged form:

(a)
The delivery tube, chute, orifice, and splash surfaces directly above the 
container receiving the food shall be designed in a manner, such as with 
barriers, baffles, or drip aprons, so that drips from condensation and splash 
are diverted from the opening of the container receiving the food;

(b)
The delivery tube, chute, or orifice shall be protected from manual contact, 
such as by being recessed;

(c)
The delivery tube or chute and orifice of the equipment used to vend 
liquid food or ice in unpackaged form to self-service consumers shall be 
designed so that the delivery tube or chute and orifice are protected from 
dust, insects, rodents, and other contamination by a self-closing door if the 
equipment is:  

(1)
Located in an outside area that does not otherwise afford the 
protection of an enclosure against the rain, windblown debris, 
insects, rodents, and other contaminants that are present in the 
environment; or

(2)
Available for self-service during hours when it is not under the 
full-time supervision of a food employee; and

(d)
The dispensing equipment actuating lever or mechanism and filling device 
of consumer self-service beverage dispensing equipment shall be designed 
to prevent contact with the lip-contact surface of glasses or cups that are 
refilled.  

1515.2
Dispensing equipment in which potentially hazardous food (time/temperature control for safety food) in a homogenous liquid form is maintained outside of the temperature control requirements as specified in § 1005.1 shall:

(a)
Be specifically designed and equipped to maintain the commercial sterility 
of aseptically packaged food in a homogenous liquid form for a specified 
duration from the time of opening the packaging within the equipment; P 
and 


(b)
Conform to the requirements for this equipment as specified in NSF/ 
ANSI 18-2006 – Manual Food and Beverage Dispensing Equipment. P
SOURCE: Final Rulemaking published at 50 DCR 4394 (June 6, 2003); as amended by Final Rulemaking published at 59 DCR 13690 (November 30, 2012).
